SECTION 09 93 10

HARDWOOD SPORT FLOOR FINISHING AND MAINTENANCE

Display hidden notes to specifier. (Don't know how? Click Here)

Copyright 2018 - 2019 ARCAT, Inc. - All rights reserved

** NOTE TO SPECIFIER ** Bona US; wood floor finishing and maintenance products.
.
This section is based on the products of Bona US, which is located at:
23 Inverness Place E. Suite 100
Englewood, CO 80112
Toll Free Tel: 800-872-5515
Tel: 303-371-1411
Fax: 303-307-5029
Email:request info (usadmin@bona.com)
Web:https://www.bona.com
[Click Here] for additional information.
At Bona we are committed to the whole lifetime of hardwood floors. We are devoted to providing solutions that keep a floor perfect during its entire life. Through our long commitment and constant search for innovation we are able to offer lasting, sustainable results for professionals and floor owners. With Bona you can expect a commitment that lasts for generations.
Today Bona is a world leading innovator with a unique system, having local presence all across the world. Since established in 1919 we are now, through subsidiaries and distributors, present in more than 50 different countries all over the world.

- GENERAL

SECTION INCLUDES

Hardwood sport floor finishing and maintenance of the following types

Paint.

RELATED SECTIONS

** NOTE TO SPECIFIER ** Delete any sections below not relevant to this project; add others as required.

Section 09 64 00 - Wood Flooring.

SUBMITTALS

Submit under provisions of Section 01 30 00 - Administrative Requirements.

Product Data: Manufacturer's data sheets on each product to be used, including:

Preparation instructions and recommendations.

Storage and handling requirements and recommendations.

Installation methods.

** NOTE TO SPECIFIER ** Delete selection samples if colors have already been selected.

Selection Samples: For each finish product specified, two complete sets of color chips representing manufacturer's full range of available colors and patterns.

Verification Samples: For each finish product specified, two samples, minimum size 6 inches (150 mm) square representing actual product, color, and patterns.

QUALITY ASSURANCE

Installer Qualifications: Trained in application of the manufacturer's floor products.

** NOTE TO SPECIFIER ** Include a mock-up if the project size and/or quality warrant taking such a precaution. The following is one example of how a mock-up on a large project might be specified. When deciding on the extent of the mock-up, consider all the major different types of work on the project.

Mock-Up: Provide a mock-up for evaluation of surface preparation techniques and application workmanship.

Finish areas designated by Architect.

Do not proceed with remaining work until workmanship, color, and sheen are approved by Architect.

Refinish mock-up area as required to produce acceptable work.

DELIVERY, STORAGE, AND HANDLING

Store products in manufacturer's unopened packaging until ready for installation.

Store and dispose of solvent-based materials, and materials used with solvent-based materials, in accordance with requirements of local authorities having jurisdiction.

PROJECT CONDITIONS

Maintain environmental conditions (temperature, humidity, and ventilation) within limits recommended by manufacturer for optimum results. Do not install products under environmental conditions outside manufacturer's absolute limits.

- PRODUCTS

MANUFACTURERS

Acceptable Manufacturer: Bona US, which is located at: 23 Inverness Place E. Suite 100; Englewood, CO 80112; Toll Free Tel: 800-872-5515; Tel: 303-371-1411; Fax: 303-307-5029; Email:request info (usadmin@bona.com); Web:https://www.bona.com

Requests for substitutions will be considered in accordance with provisions of Section 01 60 00 - Product Requirements.

PAINT

Court Sport Floor Paint:

** NOTE TO SPECIFIER ** CourtLines Sport Floor Paint is a high-solids, fast-drying, solvent-based sport floor paint, manufactured specifically for use with both waterborne and oil-modified sport floor sealers and finishes. Available in fifteen vibrant primary colors, all CourtLines paints are blendable to achieve an endless array of custom colors.

Product: Bona CourtLines Sport Floor Paint as manufactured by Bona US.

Ingredients: Alkyd resins, odorless mineral spirits, pigments, driers, titanium dioxide.

Base Colors: Red, yellow, green, lt. blue, dark blue, orange, brown, black, white, gold, maroon, purple, lt. beige, gray, NC blue. Refer to drawings and finish schedule.

Odor: Non-offending low odor.

Solids: Colors - 47-51 percent; White - 66 percent.

Flash Point: 100 degrees F (TCC).

Density: Colors - 7.6 to 7.9 lbs./gallon; White - 11.1 lbs./gallon.

US Regulatory VOC: Does not exceed 490 g/L.

Stability: 2-year shelf life in unopened container

Color: Light Blue.

Color: Red.

Color: Green.

Color: White.

Color: Black.

Color: Orange.

Color: Dark Blue.

Color: Yellow.

Color: Brown.

Color: Purple.

Color: Gold.

Color: Maroon.

Color: Grey.

Color: Light Beige.

Color: NC Blue.

** NOTE TO SPECIFIER ** Bona Sport products continue to lead the way with trends in athletic wood floors through our hybrid paint/stain and paint/sealer applications.

Tints and Blends:

Contact manufacturer.

** NOTE TO SPECIFIER ** Bona CourtLines and Bona SuperSport Paint are customizable to meet any school or facility color needs. The colors shown on the Bona website are a representation of what's possible with Bona. These are not stock colors, however, by using the search function, you can look-up the desired PMS number and review the recommended formulas. If the color you need is not listed, please contact your Bona Sale Representative or email Paint.Request@bona.com.

Color Blend Formulas. Contact your Bona Sale Representative or email Paint.Request@bona.com.

- EXECUTION

MANUFACTURER'S INSTRUCTIONS

Comply with the instructions and recommendations of the floor finish system manufacturer.

For new installation of floors, use only in structures with humidity and temperature controls. Do not use over channel and clip floor systems in environments not controlled for temperature and humidity.

EXAMINATION

Do not begin installation until substrates have been properly prepared.

If substrate preparation is the responsibility of another installer, notify Architect of unsatisfactory preparation before proceeding.

PREPARATION

Protection: Protect adjacent finish surface to prevent damage during sanding and sport floor system application.

Surface Preparation:

Sand and prepare floor using MFMA accepted methods.

Vacuum thoroughly.

APPLICATION

Specific application methods vary with product. Consult manufacturer and comply fully with manufacturer's application recommendations.

Before using these products, read and understand all directions and the Material Safety Data Sheets (MSDS).

Shake well before using.

Do not thin products.

Keep from freezing.

Finish System: As scheduled or indicated on the drawings.

PROTECTION

After application, protect floor finish from damage during subsequent work.

Do not allow foot traffic until floor is sufficiently dried and cured.

END OF SECTION